Part 1

Compute center of earth to reflector or feature vectors.

Write out vectors with time. Maybe need relativistic correction.

EOP not needed as at center of earth.

Part 2

For each reflector and feature file,

For each time

Compute lighttime offset between center of earth and station.

Retard time of interpolation of center of earth state vector by this

amount to get lunar range at the same time as c-of-e.

Subtract properly rotated station vector from state vector to get state

vector at the time of interpolation.

Do same thing with return leg.

Compute range from sqrt of sum of squares of leg, for both directions.

Detail:

Compute outgoing range

Translate from C-of-E to station

Compute outgoing range

Difference the ranges.

increase interp time by diff in range time

Computer outgoing range.

Iterate current and previous range until closure??

Do same with inbound range.

Format requirements:

- EOP

- outgoing, returning statevector (center of earth and reflector/feature

- nutation values (4)

- relativistic correction(s)

Notes on relativistic correction:

This goes as A+B+AB/(A+B-AB). Consider full moon, new moon, and quadrature.

Say the sun-earth distance (A) = 500 sec and AB = 1.25 sec

Full: B= 500+1.25, and the quotient becomes: 1.0025

New:  B= 500-1.25, and the quotient becomes: 1.002506

Quad: B= 500, and the quotient becomes: 1.002503

So the variation is 5e-6, far below needs for predictions, given an overall

relativistic effect of 25nsec, one way.

Earth-moon distance vary from 1.15 to 1.35 sec, giving values of

Full: 1.0023, 1.0027

New:  1.002305,...

Earth's orbit ellipticity (re-rm)/rm = 0.0034.

Hmmm, One day's test shows that the outbound correction goes from 23.362-

24.230 nsec. Bakleg correction is similar, giving an overall change of

1.7 nsec.

To run genpred using the center of earth coordinates:

[rlr@farmalldhcp elunpred]$ ./genpred -f PRDCTRL.PCAgeo

GENERALIZED SOLAR SYSTEM PREDICTION PACKAGE

************************

ENTER OUTPUT EPHEMERIS FILE NAME:

geocx

Enter polar motion file name (just return for "poldat")

ENTER INITIAL JULIAN DATE (MM DD YYYY HH MM SS):

7 9 2002 0 0 0

ENTER FINAL   JULIAN DATE (MM DD YYYY HH MM SS):

7 10 2002 0 0 0

Aberration:

Apparently, the aberration corrections to the point angle are of the wrong sign,

which had been suspected but not previously confirmed. Using USNO NOVAS code aberat routine, here is a comparison:

bodtel  -0.00227567855 -0.000103913452  0.0010656014 -0.000602025677

0.0137364909 -1.54521665E-05

tlrfvec   0.000475268016 -0.00222670668 -0.00106801477  0.000584151723

0.000151732782  1.24950011E-05

stvec   0.294566371 -0.892743488 -0.387050387  0.0161934171  0.00451978301

0.0019586884 DIST   0.00251490713 VLITE   173.144633

old aberr -2.35207638E-07 -6.5649361E-08 -2.84497379E-08  <==

usno aberr  2.40074449E-07  4.28535494E-08  1.75159682E-08<==

celtel   0.000506637685 -0.00222096738 -0.00106554212  0.00058227166

0.000158540402  1.54553417E-05

bodtel   0.00227564618  0.000103982348 -0.00106554212  0.000602562127

-0.0137362785  1.54553417E-05

Now regarding the MLRS acq code: it applies no aberration correction, because

it is applied to the input station to center of moon coordinates.

DTADT shouldn't have to be carried or computed in genprst. It seems to make

about .5 nsec difference in the range at the horizon and ~1 near the zenith?

Aberration: " SEE MURRAY (1981) MON. NOTICES ROYAL AST. SOCIETY 195, 639-648."

Sun velocity needed for aberration can be computed in closed form w/ usno

novas solsys3 routine.

Setting UT1C=0 in genpred resulted in error wrt standard run of 608->-624 nsec.

Current versions w/ proper UT1C give errors of 9 -> -36nsec h to h.

Problem was that the WOBBLEREC needs to be applies to BODY FIXED coordinates,

and need to be applied with negatice signs to xpole and ypole. Both USNO and

our routines give the same results. Now we are down to 0 error near the

horizon and 2 nsec difference at the meridian.