MLRS Lunar Prediction, Filtering, and Normal Point Manual

## Introduction

The McDonald Laser Ranging Stations (MLRS) developed several pieces of software to facilitate Lunar Laser Ranging (LLR). While some are MLRS-specific and of little use to anyone else, and while some are embedded into the MLRS tracking and ranging code, there are 3 pieces of software (plus 2 small specialty programs) that are of more general interest. These are the prediction program used to provide lunar reflector and center predictions to the ILRS, a Poisson filtering program used to tease the few lunar returns from a welter of noise returns, and, finally, the program to create lunar normal points from filtered lunar returns. The Poisson filtering program is used for MLRS satellite processing as well, and forms the basis for other NASA station filtering.

The biggest difference between lunar and satellite predictions and processing is the fact that lunar orbit requires working in a solar-system barycentric reference frame due to light time and relativistic effects rather than a geocentric reference frame used for most artificial earth satellites. Also, the lunar orbit is taken/interpolated from the JPL lunar and planetary ephemerides; new ephemerides are available every few years rather than daily. The O-C range residuals relative to such ephemerides are always in the range of a few nsec, making the extensive orbit fitting and time/range bias removal done in SLR normal pointing programs unnecessary.

This manual includes information about how to use these programs, and sets down the formats of all the files used, and gives examples of test runs.

Current ILRS formats are used. The output prediction files are Consolidated Prediction Format (CPF) files,[1][2] ranging data is transferred in the Consolidate Ranging data Format (CRD).[3] [4]

## Predictions

Program genpred

### Description

Computes predictions for the moon's center and all lunar retro-reflector arrays and writes them in the ILRS CPF file format.

The program could at one time use the MIT or JPL ephemeris formats, although the former has not been used for many years, and the code has been removed. Other ephemerides could probably be used with appropriate interface routines. Currently, the DE-421 ephemeris is used. Please check from time to time, as the preferred ephemeris for lunar ranging changes ever few years.

### Command Line Arguments

-a[nnn]	- automatic predictions up to 3 digit argument specifying the numbers of days in the future to predict; defaults to 3 days (moon rises) if no number is present. If this argument is not included, the user will be prompted for start and stop times and other parameters.

-p filename	- polar motion file name; 255 characters, max; located in directory lib

-f filename	- control file name; 14 characters, max; located in directory lib

-e filename	- binary ephemeris file name; 256 characters, max; located in directory pred

-r		- reverse time (i.e., go backwards in time)

-31		- output earth velocity record (record 31)

All filenames are limited to 14 characters and are expected in their default directories.

### File formats

See Appendix A

### Execution example

See prod/bin/XClungeo. This script is used to create lunar predictions found on the CDDIS and EDC web sites. Predictions are created for 5 days into the future at 15 minute (900 second) intervals.

The binary JPL ephemeris is expected in  file data/pred/jpleph. Automatically produced CPFs are found in data/pred/421 (for DE-421 ephemeris).

To manually reproduce the first day of the CPFs found in data/pred/421/ref, run genpred manually from the top level directory (above “data”, “bin”, and “src”). The poldat dat file in data/lib is used and is a copy of the poldat_011717 found in the same directory. The files in data/pred ending in “.lun_pred_log” show the predictions for all targets for each record as well as a recap of all the input parameters.

rlr@Mate-test:~/MLRS_Lunar_Code$ bin/genpred -f PRDCTRL.PCAg21

Generalized Solar System Prediction Package v1.02 12 May 2010

*******************************************

Enter output ephemeris file name:

2017_018				←User enters this

Enter polar motion file name (just return for "poldat")

←User enters this (<enter>)

ENTER INITIAL JULIAN DATE (MM DD YYYY HH MM SS):

01 18 2017 0 0 0			←User enters this

ENTER FINAL   JULIAN DATE (MM DD YYYY HH MM SS):

01 19 2017 0 0 0			←User enters this

INITIAL TIME:  2457771.5000   01/18/2017  00:00: 0.0

FINAL TIME:    2457772.5000   01/19/2017  00:00: 0.0

EPHEMERIS POINT SEPARATION:    900.

MINIMUM ELEVATION:   0.0

JPL ephemeris;  file name is:

jpleph.421

rlr@Mate-test:~/MLRS_Lunar_Code$

## Poisson Filtering

Program Poisson

### Description

Poisson provides a way to use Poisson statistics to filter the full rate lunar or satellite data.[5] It is designed to separate the wheat (lunar returns) from the chaff (noise returns) when there is substantial noise content in the range window.

The program uses ranging data converted to O-C range residuals vs time and then bins the data in residual space. O-C bins are take 2 at a time (adjacent bins), and the question is asked mathematically “How does the number of points in this bin pair compare to the maximum number expected in any bin pair given the number of bin and points in the data set?” The farther the number of returns in the bin pair exceeds the maximum statistically-expected number, the higher the likelihood that this bin contains real returns from a satellite or lunar reflector.

Because there may be a significant slope to the O-C range residuals (for SLR data, mainly), the program iterates through a range of slopes, trying to find the optimal slope. When there are multiple choices, the program reports on viable alternatives, but chooses a winner on its own.

### Command Line Arguments

-c filename	- command parameter filename (defaults to data/lib/filter_crd.pc)

-i filename	- input crd filename

-o filename	- output crd filename

-l filename	- log filename

-s filename	- summary filename

### File formats

Input configuration files:

/data/lib/filter_crd.pc

Contents of each record, one for each lunar reflector or satellite:

sic		- i.e., satellite ID code

binlength	- length of time bin (sec)

binwidth	- width of residual semi-bin (nsec)

minslope	- maximum negative slope for data (nsec/hour)

maxslope	- maximum positive slope for data(nsec/hour)

minyd	- minimum acceptable range residual (nsec)

maxyd	- maximum acceptable range residual (nsec)

psens	- fudge factor to desensitize Poisson filter (points)

minpts	- the minimum number of points in a time bin for

the bin to be accepted as data (points)

mint	- minimum time of data used in filtering (hours)

maxt	- maximum time of data used in filtering (hours)

minres	- minimum o-c range residual used in Poisson analysis (nsec)

maxres	- maximum o-c range residual used in Poisson analysis (nsec)

lunar example:

0100     3600.0     0.7500       -5.0        5.0      -30.0       30.0    0    3     0.0000   999.9999     -300.0     +300.0

0101     3600.0     0.7500       -5.0        5.0      -30.0       30.0    0    3     0.0000   999.9999     -300.0     +300.0

0102     3600.0     0.7500       -5.0        5.0      -30.0       30.0    0    3     0.0000   999.9999     -300.0     +300.0

0103     3600.0     0.7500       -5.0        5.0      -30.0       30.0    0    3     0.0000   999.9999     -300.0     +300.0

0104     3600.0     0.7500       -5.0        5.0      -30.0       30.0    0    3     0.0000   999.9999     -300.0     +300.0

Input data files:

Standard, range calibrated, CRD full rate data file

Output data file:

Standard CRD full rate data file

### Execution example

## Lun_auto_crd and Lun_sdqi_crd

### Description

Our experience at MLRS is that lunar returns can arrive in irregular clumps during the 45 minutes or less of ranging to a lunar target. To take advantage of these clumps, especially back in the days of manually editing, normal points were created based on clumps and voids, not arbitrary spans of time. (This is handled differently at other laser stations with larger apertures and a larger return rate.) Lun_auto_crd was created to automate this process, looking at clumps and voids in data and creating breaks where the data suggests them. At these points in the data span, the program writes out special, MLRS-specific “94” records which are used in the normal point program to force a break in the normal point.

The “subjective data quality indicator” (“Data quality assessment indicator” in CRD record 50) was determined by the operator reducing the data before LLR post-processing became automated. The program lun_sdqi_crd was written to automate this task for MLRS lunar data.

These programs can be bypassed if not wanted as long there is a mechanism to set the normal point length.

### Command Line Arguments

-i filename	- input crd filename

-o filename	- output crd filename

-l filename	- log filename

-s filename	- summary filename

Same arguments for both programs.

## Normal Point

Program llr_npt

### Description

As with satellite data, lunar ranges can be condensed from a few to hundreds of verified returns to a few normal points.[6] Unlike satellite data, the ranges to the lunar retro-reflector arrays (LRAs) require reference to a lunar and planetary ephemeris, directly or through a lunar CPF, to compute and iterate the outbound and inbound travel times, accounting for relativistic effects. Unlike satellite data, lunar data range residuals are essentially flat over about 45 minutes, making a polynomial fit to the O-C residuals unnecessary.

While this program could, in principle, use lunar CPFs to create the normal points, the existing normal point program predated the CPF format and has not been converted to use them.

An option for running this program (-r) just calculates the O-C range residuals for each of the filtered returns and writes them to the CRD file in special MLRS-only 93 records. The program is run with this option at the beginning of processing, providing the Poisson program with up-to-date residuals to use in filtering.

Skew, kurtosis, and peak-mean are not included in the “50” session record, but are on the “11” normal point records.

### Command Line Arguments

-r		- recalculate raw return residuals (for Poisson filtering)

-n		- compute normal points

-i filename	- process raw data in filename

-o filename	- specify output filename

-m filename	- specify mini-normal point filename (OBSOLETE)

-c filename	- output O-C residual filename

-l filename	- output log filename

-s filename	- output summary filename

-p filename	- input polar motion filename

-f filename	- input control filename

-tnnnnnn	- normal point duration, nnnnnn, in seconds, e.g. -t900

-gnn.n		- sigma for filtering, e.g., -g3, -g2.5; default is 3.0

### File formats

See Appendix A

### Execution example

See prod/bin/ldb_crd. This script is used to filter lunar data and created normal points and fully filtered full rate data. The file npt_test.sh in the top level directory can be used to run several lunar data sets. The input CRD files are in data/analysis directory ending in “.frcal”. These are fully calibrated full rate files. After running the script, other files will appear in that directory, including the normal points (.npt) and full rate data (.frd) and log files (.lsc for summary and .llc for full log).

The bin/ldb_crd script used a “poldat” file (poldat_thruJ17) that extends from 1976- January, 2017. Because the polar motion and earth rotation used in the original normal point processing differ from the observed polar motion and earth rotation in the poldat_thruJ17, the normal points may not exactly match those found on the ILRS website. The computer hardware and version of operating system used to produce the referenced output (see data/analysis/ref) may also differ from your test configuration, resulting in additional small differences.

To test the normal point processing, execute npt_test.sh in the MLRS directory as shown below.

rlr@Mate-test:~/MLRS_Lunar_Code$ ./npt_test.sh

== Started processing lunar data file s25y11d016t0231#103. ==

Recalc is now executing...

Poisson Filter is now executing...

Lun_auto is now executing...

Lun_sdqi is now executing...

NormalPoint is now executing...

Cleaning up...

== Finished processing lunar data file s25y11d016t0231#103! ==

comparing normal point files:

comparing full rate files:

### JPL Code and Ephemeris

The JPL FORTRAN code needed to create, read, and interpret the binary JPL DE-xxx ephemerides has copyrights that are in a gray area. Thus they are not included in this distribution. The reader of this manual will need to get the software themselves and make needed changes to integrate them.

- Get the documentation from: ftp://ssd.jpl.nasa.gov/pub/eph/planets/README.txt

- Get the FORTRAN code and user guide from: ftp://ssd.jpl.nasa.gov/pub/eph/planets/fortran

- Get the DE-421 ascii ephemeris from: ftp://ssd.jpl.nasa.gov/pub/eph/planets/ascii/

- Create the binary DE-421 ephemeris according to the user guide instructions.

- Both genpred and llr_npt need a modified version of testeph.f.

- Copy testeph.f into src/genpred and run from the command line:

patch testeph.f testeph.diff

- Rename the patched testeph.f to testeph.F .

- Copy testeph.F to llr_npt.

- Run “make” in both directories.

## Earth Orientation Download

Program BullA

### Description

Program BullA and its execution script XCgetBullAM down loads USNO Bulletin A in the Mark3 format and reformats it for us with the lunar programs. The output file is the poldat* file found in the data/lib directory. (There is a poldat_thruJ17 file in this directory that can be used for any processing before January 2017. It is used in the ldb_crd script for creating sample output.)

### Poldat format:

Header Record:

F10.1 – Start Julian Date

F10.1 – End Julian Date

F3.1  - Time between entries (days)

Example:  2457024.5 2457176.5 1.0

Data Record:

F10.1 – Julian Date

F7.4 – X pole

F7.4 – Y pole

F8.5 – dUTC

Example:  2457024.5  0.0296  0.2812 -0.46083

## File Tree

Directories in the distribution follow the production files tree. It is laid out as follows on Unix/Linux systems. These are all in reference to the directory from with the programs are executed.

adm	- cron files needed for production

bin	- the executable programs and scripts

data/analysis	- output from the processing steps, including normal points and distributable full rate       data, processing summaries, and by-products.

data/lib	- parameter files needed to run the programs

data/log	- log files from BullA downloads

data/pred	- binary JPL ephemeris input to prediction and normal pointing steps and reports and daily CPF files.

src	- directories with the source files.

## Final Comments

MLRS lunar software was originally developed in the late 1970s and has been updated, maintained, and improved through the years. The core lunar prediction software was developed by Richard Abbot, then a graduate student in the UT Astronomy Department. The bulk of additional software development was conducted by Randall Ricklefs of UT McDonald Observatory and Center for Space Research. Maintenance for a number of years was assisted by Thomas Riley, then employed by the UT McDonald Observatory.

The source code was compiled under Ubuntu 16.04, and the binaries and scripts in “bin” should work on similar vintage Linux distributions. To compile the entire suite of software (once testeph has been downloaded and modified), go to src and execute “./make.sh ”

If you find this software useful and make significant improvements, please make these available to other users. This software was written under numerous NASA grants and contracts. Permission has been given by NASA for UT to open source this code under the BSD3 license. The code is (c) 2017, The University of Texas at Austin. Please see Appendix D for the full license.

Appendix A – Lunar parameter files

LCONTROL.PCA

Format:

Lines starting in '0'

contain the names of the parameter sets found with lines starting with 1-9.

Lines starting with 1-9 contain prediction or normal point control sets.

Appropriate versions of each are copied to the NPTCTRL.PCA and PRDCTRL.PCA files.

First line:

name of control set – usually the same as in the '0' lines.

Second line:

10 T/F options for each entry in the LSITES.PCA file.

5 lunar retro reflector arrays: T/F, all can be chosen.

Third line:

model parameter set chosen (1-10)

start and stop Julian dates of computations. Usually overwritten via the command line.

Separation of predictions in seconds: usually 900

Minimum elevation of predictions in degrees

Fourth line:

20 debug flags for extra printing: T/F

Fifth line:

reflector coordinate set (1-10) from LREFL.PCA

ephemeris type: 0= mit, 1= jpl

ephemeris file name, e.g. jpleph

Example:

0 MIT

0 JPL

0 GEOCENTER

0 TEST

0 GEOCENTER DE-421

0 MLRS DE-421

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

1 MIT

1  T F F F F F F F F F      T T T T T

1        1  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

1  F F F F F F F F F F F F F F F F F F F F

1         1        0 miteph

2 JPL

2  F T F F F F F F F F      T T T T T

2        2  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.10000000E+02

2  F F F F F F F F F F F F F F F F F F F F

2         2        1 jpleph

3 GEOCENTER - mit

3  F F T F F F F F F F      T T T T T

3        1  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

3  F F F T T F F F F F F F F F F F F F F F

3         1        0 miteph

4 GEOCENTER - JPL

4  F F T F F F F F F F      T T T T T

4        1  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

4  F F F T T F F F F F F F F F F F F F F F

4         2        1 jpleph

5 TEST

5  F F F T F F F F F F      T T T T T

5        1  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

5  F F F F F F F F F F F F F F F F F F F F

5         1        0 miteph

6 GEOCENTER - JPL-421

6  F F T F F F F F F F      T T T T T

6        3  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

6  F F F T T F F F F F F F F F F F F F F F

6         3        1 jpleph.421

7 MLRS - JPL-421

7  F F F F F F T F F F      T T T T T

7        3  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.00000000E+00

7  F F F T T F F F F F F F F F F F F F F F

7         3        1 jpleph.421

LMODEL.PCA

Format:

Lines starting in '0':

contain the names of the parameter sets found with lines starting with 1-9.

First line:

name of control set – usually the same as in the '0' lines.

Second line:

Two way predictions? T/F

Use polar motion file? T/F

Tolerance of inbound and outbound leg closure: e.g.: 0.1e-13

Maximum number of iterations of inbound and outbound legs: e.g.: 10

Second line:

Speed of line in km/sec

Astronomical Units per km

Earth:Moon mass ratio

Third line:

Lunar K2 Love number

Polar Motion source title

Include UT1 tide? T/F

Example:

0 MIT

0 JPL

0 JPL-421

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

1 MIT

1  T T  0.100E-13    10

1   0.299792458000E+06   0.149597870650E+09   0.813005979100E+02

1  0.290E+00 NOT DEFINED . . .   F

2 JPL

2  T T  0.100E-13    10

2   0.299792458000E+06   0.149597870691E+09   0.813005560100E+02

2  0.290E+00 NOT DEFINED . . .   F

3 jpl_de-421

3  T T  0.100E-13    10

3   0.299792458000E+06   0.149597870691E+09   0.813005690700E+02

3  0.216+00 NOT DEFINED . . .   F

LREFL.PCA

Format:

Lines starting in '0':

contain the names of the parameter sets found with lines starting with 1-9.

First line:

name of control set – usually the same as in the '0' lines.

Second line:

Reflector 0 (Apollo 11; Tranquility) selenographic coordinates:

Radius (km), Latitude (deg), Longitude (deg)

Third line:

Reflector 1 (Lunakhod 1; Luna 17) selenographic coordinates:

Radius (km), Latitude (deg), Longitude (deg)

Fourth line:

Reflector 2 (Apollo 14; Fra Mauro)selenographic coordinates:

Radius (km), Latitude (deg), Longitude (deg)

Fifth line:

Reflector 3 Apollo 15; Hadley) selenographic coordinates:

Radius (km), Latitude (deg), Longitude (deg)

Sixth line:

Reflector 4 (Lunakhod 2; Luna 21) selenographic coordinates:

Radius (km), Latitude (deg), Longitude (deg)

Example:

0 MIT

0 JPL DE-405

0 JPL DE-421

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

1 MIT

1  1735.475309808      0.69329630     23.45167049

1  1735.000000000     38.30500000    -35.21400000

1  1736.339452989     -3.62342011    -17.49971649

1  1735.479783667     26.15502212      3.60779843

1  1734.641340616     25.85082867     30.90612720

2 JPL DE-403

2  1735.472177         0.6935059      23.4552875

2  1735.000000000     38.30500000    -35.2140000

2  1736.335386        -3.6232501     -17.4961335

2  1735.476394        26.1552499      3.6113259

2  1734.638189        25.8510531     30.9097439

3 JPL DE-421

3  1735.472732         0.6934309      23.4543075

3  1734.929200        38.3330890     -35.0366860

3  1736.336135        -3.6233288     -17.4971065

3  1735.477340        26.1551743       3.6103520

3  1734.639009        25.8509928      30.9087426

LSITES.PCA

Format:

Lines starting in '0':

contain the names of the parameter sets found with lines starting with 1-9.

First line:

name of observing site set – usually the same as in the '0' lines.

Second Line:

Transmitting Telescope coordinates:

Earth radius at site in km

Geocentric latitude in degrees

East longitude in degrees

Geodetic latitude in degrees

Third Line:

Receiving Telescope coordinates:

Earth radius at site in km

Geocentric latitude in degrees

East longitude in degrees

Geodetic latitude in degrees

Note: In most cases, the transmit and receive telescope are the same. That was not the case at Haleakala.

Example:

0 MLRS (MIT) (71112)

0 MLRS (JPL) (71112)

0 GEOCENTER (0)

0 MLRS (JPL-DE-421)

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

0 NOT CREATED

1   71112. MLRS (MIT) (71112)

1   6374.60990057     30.51171119    255.98478220     30.68026700

1   6374.60990057     30.51171119    255.98478220     30.68026700

2   71112. MLRS (JPL) (71112)

2   6374.60983000     30.51171089    255.98480380     30.68026674

2   6374.60983000     30.51171089    255.98480380     30.68026674

3      0. GEOCENTER (0)

3      0.00000100      0.00000000      0.00000000      0.00000000

3      0.00000100      0.00000000      0.00000000      0.00000000

4   71112. MLRS (JPL-421) (71112)

4   6374.60983000     30.51171089    255.98480380     30.68026674

4   6374.60983000     30.51171089    255.98480380     30.68026674

NPTCTRL.PCA

Format:

Same as for control parameters. Used to control normal point creation.

1 JPL_DE-421

1  F F F F F F T F F F      T T T T T

1        3  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.10000000E+02

1  F F F F F F F F F F F F F F F F F F F F

1         3        1 jpleph

PRDCTRL.PCA

Format:

Same as for control parameters. Used to control prediction creation.

1 JPL

1  F T F F F F F F F F      T T T T T

1        2  0.24457246E+07  0.24457250E+07  0.90000000E+03  0.10000000E+00

1  F F F F F F F F F F F F F F F F F F F F

1         2        1 jpleph

Appendix B – CUTABLE leap second file

CUTABLE is used in genpred and llr_npt to handle leap seconds. The file is read by subroutine ephinit and used in subroutine cutable. The entries are the Julian date (JD) of the leap second and the value of Atomic Time (AT) – UTC after the leap second at the JD, and the rate of AT-UTC sec/day.  The rate has been set to 0.0 for every leap second since JD 2441317.50. The formula is:

AT - UTC = AT-UTC at JD0 + RATE*(JDcurrent-JD0)

B.1 Format

3X

F10.2	– Start Julian date (JD0) in days

“ ,  “

F11.8	- AT-UTC at JD0 in seconds

“ ,  “

F11.2	- Rate of AT-UTC in seconds/day

The last record in the file is always:

“   2600000.50 ,    .00000000 ,          .00”

Appendix C - MLRS 9x record additions for lunar data processing

There are several more MLRS 9x records. Those below are used by lunar processing programs.

C.1 “93” Intermediate Range Information Record

C.1.1 Format

A2(1-2)	Record Type (= “93”)

F18.12	Seconds of day

A4	System configuration ID

I1	Full range type indicator

F7.3	K (fire) calibration value (nsec)

F7.3	K' (return) calibration value (nsec)

F7.3	Geometric correction (eccentricity and MLRS optical lever arm) (nsec)

F9.5	Range refraction (nsec?)

F9.5	Elevation refraction (radians)

F11.3	Range O-C residual (nsec)

F11.3	Range O-C residual – post fit (nsec)

C.2 “93” Normal Point Break Record

C.2.1 Format

A2(1-2)	Record Type (= “94”)

F9.3	Seconds of day of a normal point break

C.3 “95” MLRS Software Versions Record

C.3.1 Format

A2(1-2)	Record Type (= “95”)

A10	Monitor Program Version

A10	Satellite Tracking Program Version

A10	Decode Post-processing Program Version

A10	Internal Calibration Post-processing Program Version

A10	Poisson Program Version

A10	Quick Look Engineering Data Program Version

A10	Normal Point Program Version

A10	Full-rate Data Stripping Program Version

Appendix D – BSD3 License for MLRS Lunar Software

All software in this distribution is...

Copyright (c) 2017, The University of Texas at Austin

All rights reserved.

Redistribution and use in source and binary forms, with or without modification, are permitted provided that the following conditions are met:

1. Redistributions of source code must retain the above copyright notice, this list of conditions and the following disclaimer.

2. Redistributions in binary form must reproduce the above copyright notice, this list of conditions and the following disclaimer in the documentation and/or other materials provided with the distribution.

3. Neither the name of the copyright holder nor the names of its contributors may be used to endorse or promote products derived from this software without specific prior written permission.

THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

References:

[1] Randall Ricklefs ILRS Prediction Format Working Group, Creating a Consolidated Laser Ranging Prediction Format, 13th International Workshop on Laser Ranging, Washington, October, 2002.

[2] http://ilrs.gsfc.nasa.gov/data_and_products/formats/cpf.html

[3] R Ricklefs, Consolidated Laser Prediction and Data Formats: Supporting New Technology, 15th International Workshop on Laser Ranging, Canberra, October, 2006.

[4] http://ilrs.gsfc.nasa.gov/data_and_products/formats/crd.html

[5] Ricklefs, R., Shelus, P., "Poisson Filtering of Laser Ranging Data", 8th International Workshop on Laser Ranging Instrumentation, Annapolis, May, 1992, p. 9-26.

[6] Abbot, R. I., P. J. Shelus, J. D. Mulholland and E. C. Silverberg, Laser Observations of the Moon: Identification and construction of normal points for 1969-1971, Astron. Jour., 78, 784-793, 1973.

Randall L. Ricklefs, MLRS, McDonald Observatory, The University of Texas

February, 2017